Module  8 :  CCNA – Network Access Basic Routing and Advance routing concepts, Switching concepts

- Explain Switch ?

- It’s use to connect multiple devices like computer, printer, and server in a network.

- It use data move from one device to the correct destination.

- In a LAN, a switch uses a MAC address to decide where to send data, each device connected to the switch has a unique MAC address.

- It’s improves network speed and reduces unnecessary traffic.

- Explain Switch Boot Sequence ?

- POST : - First of all switch checks its hardware to ensure everything is working properly.

- Then switch loads its software IOS from memory.

- The switch loads the saved settings.
If no settings are saved, it uses default settings. Then it’s ready to send and receive data.

- Explain Three Methods to access switch Command line interface.

- I. Console Access :

- It is used to directly connect a computer to a switch or router using console cable.

- It allows direct access to the router’s user mode without a password and is used for initial configuration.

Command :

Switch> enable

Switch# configure terminal

II. Telnet Access :  work on Port =23

- Telnet allows remote access to the switch over the network. It’s not secure because data is sent in plain text.

Command :

Router> enable

Router# configure terminal

Router(config)# line vty 0 4

Router(config-line)# password cisco

Router(config-line)# login

Router(config-line)# exit

Access router use telnet from pc

telnet [ip address of pc ]

III. SSH Access Method :Use port 22

- It’s secure remote access method used to manage router & switch.

- More secure than telnet & use to access cisco ios device remotely.

Command :

Router> enable

Router# configure terminal

Router(config)# hostname Router1

Router1(config)# ip domain-name network.com

Router1(config)# username admin password cisco

Router1(config)# crypto key generate rsa

Router1(config)# line vty 0 4

Router1(config-line)# login local

Router1(config-line)# transport input ssh

Router1(config-line)# exit

- Explain and Configuring the Cisco Internet Operating System.

Ans :It’s use to Operating system for Cisco routers & switches.

- It controls networking, security & device Management.

- It supports protocols like RIP, OSPF , EIGRP, BGP, VLANS

- Use Network services like DHCP, NAT

Command :

Router> enable

Router# configure terminal

Router(config)# hostname R1

Set password (console ,vty , enable secret).

- Explain Switch  Port ?

- It’s a physical interface on a network switch used to  connect  devices  such as computers, printers, servers or other switches to a local area network (LAN).

- It operates at Data Link Layer (layer 2) & forward data on MAC addresses.

- IT is to reduce collision.

- There are multiple types of switch port :

- Access port

- Connect end devices , carries traffic for one VLAN.

- Trunk Port

- Connect switches or network devices , carries traffic for multiple VLANS.

Q 6 -R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two)

Ans :  R1, R2

7. Enable secret [password] is hashed using the algorithm .

A. MD5 				B. AH

C. PSK 				D. ESP 		E. WPA2

Ans : A. MD5

8.  An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A. R1 is an Area Border Router.  			B. R1 is a backup designated router.

C. Router 2.2.2.2 is an

Area Border Router. 				D. Router 2.2.2.2 is a backup designated router.

Ans :  D. Router 2.2.2.2 is a backup designated router.

9. Which command is used to view the neighbor discovery table on a PC?

A. show ipv6 neighbor                       B. show ipv6 neighbors

C. netsh interface ipv6

show neighbor 				D. netsh interface ipv6 show neighbours

Ans : D. netsh interface ipv6

10. What type of variable is being shown? Routers = [R1,R2,R3]

A. List 						B. Dictionary

C. Simple 					D. Unsigned integers

Ans :  A. List

11.  Identify the fields in an IPv4 header. (Choose three)

A. Host component 			B. Time to Live

C. Source address 			D. Destination address

Ans :

B. Time to Live

C. Source address

D. Destination address